CMPG325 - Milestone 2 Configuration Summary

Project ID: CMPG325-2026-034

Client ID: CLI-034

Addressing Block: 172.30.16.0/23

Network Architecture: Extended-Star / Hierarchical

Network Device Inventory

| Device | Role | Main Function |
| --- | --- | --- |
| CORE-SWITCH | Layer-3 Core | Inter-VLAN routing and DHCP |
| SW-GROUND | Access switch | Administration and Sales |
| SW-ENG | Access switch | Engineering |
| SW-PROD | Access switch | Production |
| SW-WARE | Access switch | Warehouse |
| SW-SERVER | Access/server switch | Server VLAN and HTTP service |
| SW-NEWFLOOR | CR2 access switch | New Floor / CR2 |

VLAN Configuration

| VLAN | Name | Network | Gateway |
| --- | --- | --- | --- |
| 10 | Administration | 172.30.16.0/26 | 172.30.16.1 |
| 20 | Sales | 172.30.16.64/26 | 172.30.16.65 |
| 30 | Engineering | 172.30.16.128/26 | 172.30.16.129 |
| 40 | Production | 172.30.16.192/26 | 172.30.16.193 |
| 50 | Warehouse | 172.30.17.0/26 | 172.30.17.1 |
| 60 | Servers | 172.30.17.64/27 | 172.30.17.65 |
| 70 | New Floor / CR2 | 172.30.17.96/27 | 172.30.17.97 |
| 99 | Management | 172.30.17.224/28 | 172.30.17.225 |

Key Services

| Service | Device | Configuration |
| --- | --- | --- |
| Inter-VLAN Routing | CORE-SWITCH | Layer-3 SVIs |
| DHCP | CORE-SWITCH | Centralised DHCP pools |
| HTTP | SRV-HTTP | Internal web service |
| Server IP | SRV-HTTP | 172.30.17.66/27 |
| Server Gateway | SRV-HTTP | 172.30.17.65 |
| Management | All switches | VLAN 99 |
| EtherChannel | CORE-SWITCH ↔ SW-SERVER | Port-Channel 1 |
| EtherChannel Protocol | CORE-SWITCH ↔ SW-SERVER | LACP |

# HTTP Server Configuration

- Device: SRV-HTTP

- VLAN: 60

- IP Address: 172.30.17.66

- Subnet Mask: 255.255.255.224

- Default Gateway: 172.30.17.65

- Service: HTTP

- Status: Enabled

- Web Page: index.html

# The HTTP server provides the assigned internal web service for the project. The server is placed in the dedicated Server VLAN 60 and uses a static IP address so that clients can consistently access the service. The Milestone 1 design places HTTP/DHCP services in the dedicated server segment.

# DHCP Configuration

# DHCP is provided centrally by CORE-SWITCH. The client VLANs receive addresses dynamically while the HTTP server remains statically configured.

| VLAN | DHCP Network | Default Gateway |
| --- | --- | --- |
| 10 | 172.30.16.0/26 | 172.30.16.1 |
| 20 | 172.30.16.64/26 | 172.30.16.65 |
| 30 | 172.30.16.128/26 | 172.30.16.129 |
| 40 | 172.30.16.192/26 | 172.30.16.193 |
| 50 | 172.30.17.0/26 | 172.30.17.1 |
| 70 | 172.30.17.96/27 | 172.30.17.97 |

# LACP EtherChannel Configuration

# Port-Channel: Po1

# Protocol: LACP

# CORE-SWITCH:

# Fa0/6

# Fa0/7

# SW-SERVER:

# Fa0/23

# Fa0/24

# Mode:

# LACP active

# Type:

# Layer-2 trunk

# Native VLAN:

# 99

# Allowed VLANs:

# 60,99

Two physical links between CORE-SWITCH and SW-SERVER are bundled into Port-Channel 1 using LACP. This provides a logical trunk with redundant physical connectivity for the server segment.

# Management VLAN

# VLAN 99 is used as the management VLAN for the access switches.

| Device | Management IP |
| --- | --- |
| CORE-SWITCH | 172.30.17.225 |
| SW-GROUND | 172.30.17.226 |
| SW-ENG | 172.30.17.227 |
| SW-PROD | 172.30.17.228 |
| SW-WARE | 172.30.17.229 |
| SW-SERVER | 172.30.17.230 |
| SW-NEWFLOOR | 172.30.17.231 |

# Trunk Configuration

| Uplink | VLANs Allowed |
| --- | --- |
| CORE → SW-GROUND | 10,20,99 |
| CORE → SW-ENG | 30,99 |
| CORE → SW-PROD | 40,99 |
| CORE → SW-WARE | 50,99 |
| CORE → SW-NEWFLOOR | 70,99 |
| CORE → SW-SERVER Po1 | 60,99 |

# The access switches use 802.1Q trunk uplinks to the Layer-3 Core. Only the VLANs required by each access area and VLAN 99 for management are permitted on the respective trunks.

# DHCP Test Results

| Device | VLAN | Assigned IP |
| --- | --- | --- |
| PC-ADMIN | 10 | 172.30.16.2 |
| PC-SALES | 20 | 172.30.16.66 |
| PC-ENGINEERING | 30 | 172.30.16.130 |
| PC-PRODUCTION | 40 | 172.30.16.194 |
| PC-WAREHOUSE | 50 | 172.30.17.2 |
| PC-CR2 | 70 | 172.30.17.98 |
| SRV-HTTP | 60 | 172.30.17.66 Static |

# Verification Commands

show vlan brief

Purpose: Verifies VLAN creation.

show ip interface brief

Purpose: Verifies SVI IP addresses and operational status

show interfaces trunk

Purpose: Verifies 802.1Q trunking and permitted VLANs.

show etherchannel summary

Purpose: Verifies LACP EtherChannel membership and status.

show ip dhcp binding

Purpose: Verifies DHCP address leases.

show ip route

Purpose: Verifies the Core routing table

ping <destination>

Purpose: Verifies network connectivity.

Implementation Status

| Feature | Status |
| --- | --- |
| VLAN segmentation | Implemented |
| Layer-3 SVIs | Implemented |
| Inter-VLAN routing | Implemented |
| DHCP | Implemented |
| HTTP server | Implemented |
| 802.1Q trunking | Implemented |
| LACP EtherChannel | Implemented |
| VLAN 99 management | Implemented |
| CR2 VLAN 70 | Implemented |
| Testing | Complete |